NOTE:

- Text in Red and highlighted  in yellow and green needs confirmation/validity checks

- As rule references are prone to numbering changes and needs to be manually modified, kindly ignore highlighted in Yellow rule references. They can be linked during finalization once the draft document is frozen.

Contents

1.	Short title and commencement	4

2.	Definitions	4

3.	Application and Implementation	4

4.	Control of the Transfer of Harmful Aquatic Organisms and Pathogens through Ships' Ballast Water and Sediments	5

5.	Sediment Reception Facilities	6

6.	Ballast Water Management Plan	6

7.	Ballast Water Record Book	6

8.	Survey and certification	6

9.	Issue or endorsement of an International Ballast Water Management Certificate or Indian Ballast Water Management Certificate	6

10.	Issue or Endorsement of an International Ballast Water Management Certificate by Indian administration or administration of another country which is party to Convention	6

11.	Validity of an International Ballast Water Management Certificate or Indian Ballast Water Management Certificate	6

12.	Inspection of Ships	7

13.	Detection of Violations and Control of Ships	7

14.	Exceptions	7

15.	Exemptions	7

16.	Equivalent compliance	7

17.	Procedure for maintenance of ballast water record book	8

18.	Fees	8

19.	Penalty	8

Annex	9

Regulations for the Control and Management of Ships' Ballast Water and Sediments	9

Section A - General Provisions	9

Regulation A-1: Definitions	9

Regulation A-2: General Applicability	9

Regulation A-3: Exceptions	10

Regulation A-4: Exemptions	10

Regulation A-5: equivalent compliance	10

SECTION B - MANAGEMENT AND CONTROL REQUIREMENTS FOR SHIPS	11

Regulation B-1: ballast water management plan	11

Regulation B-2: Ballast water record book	11

Regulation B-3: Ballast Water Management for Ships	11

Regulation B-4: Ballast Water Exchange	12

Regulation B-5: Sediment Management for Ships	12

Regulation B-6: Duties of Officers and Crew	13

SECTION C - SPECIAL REQUIREMENTS IN CERTAIN AREAS	14

Regulation C-1: Additional Measures	14

Regulation C-2: Warnings Concerning Ballast Water Uptake in Certain Areas and Related Flag State Measures	14

SECTION D - STANDARDS FOR BALLAST WATER MANAGEMENT	15

Regulation D-1: Ballast Water Exchange Standard	15

Regulation D-2: Ballast Water Performance Standard	15

Regulation D-3: Approval requirements for Ballast Water Management systems	15

Regulation D-4: Prototype Ballast Water Treatment Technologies	15

SECTION E - SURVEY AND CERTIFICATION REQUIREMENTS FOR BALLAST WATER MANAGEMENT	17

Regulation E-1: Surveys	17

Regulation E-2: Issue or endorsement of an International Ballast Water Management Certificate or Indian Ballast Water Management Certificate	18

Regulation E-3: Issue or Endorsement of an International Ballast Water Management Certificate by Director General or another country which is party to Convention	18

Regulation E-4: Form of the Certificate	18

Regulation E-5: Duration and Validity of the Certificate	18

SCHEDULE 1	21

INTERNATIONAL BALLAST WATER MANAGEMENT CERTIFICATE	21

SCHEDULE 2	29

INDIANBALLAST WATER MANAGEMENTCERTIFICATE	29

SCHEDULE 3	17

Form of ballast water record book	17

SCHEDULE 4	22

Fees	22

MINISTRY OF SHIPPING

NOTIFICATION

New Delhi, the XX  Month, 2026

G.S.R.XX (X).—In exercise of the powers conferred by section XXX of the Merchant Shipping Act, XXXX (XX of XXXX),the Central Government there by makes the following rules, namely:—

### Short title and commencement

These rules may be called the Merchant Shipping (Control and Management of Ships’ Ballast Water and Sediments) Rules, 2026.

They shall come into force on the date of their publication in the Official Gazette.

### Definitions

In these rules, unless the context otherwise requires,—

- “Act” means the Merchant Shipping Act, XXXX(XX of XXXX);

- "authority", in relation to-

- (i) a ship operating under the authority of the Government of India, means the Central Government; or

- (ii) a ship entitled to fly a flag of any other Country, means the Government of that Country; or

- (iii) a floating platform engaged in exploration and exploitation of the sea-bed and sub-soil thereof adjacent to the Indian coast over which Government of India exercises sovereign rights for the purposes of exploration and exploitation of its natural resources, including Floating Storage Units and Floating Production Storage and Offloading Units, means the Government of India;

"Ballast Water" means water with its suspended matter taken on board a ship to control trim, list, draught, stability or stresses of the ship.

"Ballast Water Management" means mechanical, physical, chemical, and biological processes, either singularly or in combination, to remove, render harmless, or avoid the uptake or discharge of Harmful Aquatic Organisms and Pathogens within Ballast Water and Sediments.

"Certificate" means the International Ballast Water Management Certificate or, as the case may be, the Indian Ballast Water Management Certificate issued by Director General or organization recognized by it.

"Gross tonnage" means the gross tonnage calculated in accordance with the tonnage measurement regulations contained in Annex I to the International Convention on Tonnage Measurement of Ships, 1969 or any successor Convention.

"Harmful Aquatic Organisms and Pathogens" means aquatic organisms or pathogens which, if introduced into the sea including estuaries, or into fresh water sources, may create hazards to the environment, human health, property or resources, impair biological diversity or interfere with other legitimate uses of such areas.

"Sediments" means matter settled out of Ballast Water within a ship.

"Ship" means a vessel of any type whatsoever operating in the aquatic environment and includes submersibles, floating craft, floating platforms, FSUs and FPSOs.

“Convention” shall have the same meaning assigned to it in clause (d) of section XXX of the Act;

“Recognized organization” means any organization recognized by the Director General;

Words and expressions used in these rules but not defined and defined in the Act shall have the respective meanings assigned to them in the Act.

### Application and Implementation

- These rules are applicable to-

  - every Indian Flag Ship wherever it may be;

  - Ships not entitled to fly the flag of India, but which operate under the authority of India;

  - Ships, other than those referred to in sub rule (1). a or (1). b above, that enter a port, shipyard, or offshore or a place in India or within the territorial waters of India or any marine areas adjacent thereto over which India has, or may have, exclusive jurisdiction with respect to control of pollution under the Territorial Waters, Continental Shelf, Exclusive Economic Zone and other Maritime Zones Act, 1976, or any other law for the time being in force.

- These rules are not applicable to-

  - ships not designed or constructed to carry Ballast Water;

  - ships operating in Indian waters only, and the discharge of Ballast Water from such ships would not impair or damage the environment, human health, property or resources;

  - any warship, naval auxiliary or other ship owned or operated by Central or State government and used, for the time being, only on government non-commercial service. However, appropriate measures may be adopted so far as is reasonable and practicable without impairing operations or operational capabilities of such ships, by Government of India, as may be notified separately;

  - Permanent Ballast Water in sealed tanks on ships, that is not subject to discharge.

- All ships to which these rules apply shall be required

  - to carry the following documents on board from 8th September 2017:

    - Approved ballast water management plan (Regulation B-1).

    - Ballast water record book (Regulation B-2).

    - An International Ballast Water Management Certificate, as applicable (issued after survey under Regulation E-1).

  - to carry out Ballast Water Management in accordance with Regulation B-3 of the Annex and  be fitted with a Ballast Water Management System (BWMS), meeting the performance standards  as per Regulation D-2of the Annex, which has been type approved by a Flag Administration or a Recognised Organisation on its behalf based on the standards and test methods developed by IMO The implementation schedule for fitment of BWMS (RegulationD-2), is as follows:

| Keel laid | Implementation deadline for BWMS in order to meet Regulation D-2 |
| --- | --- |
| Before 8 September 2017 | By the first renewal survey associated with the IOPP Certificate after 8 September 2O17. |
| On or after 8 September 2017 | By the completion date of the construction |

### Control of the Transfer of Harmful Aquatic Organisms and Pathogens through Ships' Ballast Water and Sediments

- Every Indian ship and other ships which are not entitled to fly the Indian flag but which operate under the authority of Government of India, shall comply with the requirements set forth in sub-rule (2) including the applicable standards and requirements as prescribed from time to time as well as effective measures to ensure that such ships comply with the requirements, as may be specified by the Director General from time to time.

- All other vessels to which these rules apply, shall comply with such ballast water management programme as may be prescribed. The applicable control measures referred to in sub rule (1) of these rules shall be as provided in annex of these rules.

### Sediment Reception Facilities

- All ports in India, calling ships to which these rules apply, should provide facilities for sediment reception as per the guidelines to the Convention, under the surveillance, supervision, guidance and instructions of that port authority.

- Where there are no sediment reception facilities at any port in India or that the facilities available at such port are not adequate for enabling ships calling at such port to comply with the requirements of the Convention, then, the port authority in charge of such port shall provide or arrange for the provision of such sediment reception facilities in a time bound manner. The Director-General may, by notification in the Official Gazette, specify the ports in India having sediment reception facilities in accordance with the requirements of the Convention

- A port authority providing sediment reception facilities or a person providing such facilities by arrangement with the port authority, may charge for the use of those facilities at such rates and may impose such conditions in respect of the use thereof as the Director-General may, by notification in the Official Gazette, specify in this behalf.

### Ballast Water Management Plan

- Every ship, to which these rules apply, shall have approved Ballast Water Management plan on board. Such a plan shall have approval by the administration or by the organization recognized by it. The Ballast Water Management plan shall be specific to each ship and shall contain minimum information as specified in Regulation B-1 of the annex.

### Ballast Water Record Book

- Every ship to which these rules apply shall have on board a Ballast Water record book, addressing the requirements of Regulation B-2 of the annex and containing the minimum information as specified in Schedule 3.

### Survey and certification

- Every ship to which these rules apply shall be subject to survey and certification in accordance with the procedures specified in regulations of Section-E of the Annex.

### Issue or endorsement of an International Ballast Water Management Certificate or Indian Ballast Water Management Certificate

- The Director General or an organization recognized by it, shall, after the successful completion of a survey in accordance with rule 8, issue or endorse the certificate considering requirements of regulation E of the annex.

### Issue or Endorsement of an International Ballast Water Management Certificate by Indian administration or administration of another country which is party to Convention

- The Indian Administration or the Administration of other country which is party to Convention shall, after successful completion of survey and satisfaction to compliance with provisions of Convention, issue or endorse the International Ballast Water Management Certificate, considering regulation E-3 of the annex.

### Validity of an International Ballast Water Management Certificate or Indian Ballast Water Management Certificate

- The International Ballast Water Management Certificate or the Indian Ballast Water Management Certificate, as the case may be, shall cease to be valid in the following cases, namely:-

  - if the Ballast Water Management plan is changed or replaced and the certificate is not endorsed in accordance with the provisions of these rules; and

  - up on transfer of the ship to the flag of another State.

- The issue of a new certificate to a ship transferred from another flag shall be based on a valid certificate issued by the previous flag.

### Inspection of Ships

- A ship to which the Convention applies may, in any port or offshore terminal within the jurisdiction of Government of India, be subject to inspection by Surveyors authorized by the Director General of Shipping for determining whether the ship is in compliance with this rule. Except as provided in sub rule (2)of this rule, any such inspection is limited to:

  - verifying that there is onboard a valid Certificate, which, if valid shall be accepted; and

  - inspection of the Ballast Water record book, and/or

  - a sampling of the ship's Ballast Water, carried out in accordance with the guidelines developed by the IMO. However, the time required to analyse the samples shall not be used as a basis for unduly delaying the operation, movement or departure of the ship.

- Where a ship does not carry a valid Certificate or there are clear grounds for believing that:

  - the condition of the ship or its equipment does not correspond substantially with the particulars of the Certificate; or

  - the master or the crew are not familiar with essential shipboard procedures relating to Ballast Water Management, or have not implemented such procedures;

  - a detailed inspection may be carried out.

### Detection of Violations and Control of Ships

- Any foreign ship to which the Convention applies and operating within the jurisdiction of Government of India, if detected to have violated the provisions of this rule, surveyors authorized by Director General of Shipping may take steps to warn, detain, or exclude the ship and may also grant such a ship permission to leave the port or offshore terminal for the purpose of discharging Ballast Water or proceeding to the nearest appropriate repair yard or reception facility available, subject to doing so does not present a threat of harm to the environment, human health, property or resources.

- If the results of the sampling described in rule 12(1)c indicates that the ship poses a threat to the environment, human health, property or resources, then such ship shall be prohibited from discharging Ballast Water until the removal of threat identified in sampling report.

- The surveyors authorized by the Director General of Shipping may also inspect a ship when operating within the ports or offshore terminals under jurisdiction of government of India, if a request for an investigation is received from any member which is party to Convention, together with sufficient evidence that a ship is operating or has operated in violation of a provision the Convention. The report of such investigation shall be sent to the Party requesting it and to the competent authority of the Administration of the ship concerned so that appropriate action may be taken.

### Exceptions

The requirements of Regulation B-3 of the Annex shall not apply subject to the ship meeting the conditions as stated in Regulation A-3 of the Annex.

### Exemptions

The Director General, at the request of ship owner or company may grant exemption to any requirements of this rule based on provisions as specified in regulation A-4 of the Annex.

### Equivalent compliance

Equivalent compliance provisions will be as specified in Regulation A-5 of the Annex.

### Procedure for maintenance of ballast water record book

- Ship to which this rule applies shall maintain, onboard a record of ballast water operations in such a manner as prescribed having regard to the provisions of the Convention. Such record can be an electronic record system, or that may be integrated into another record book or system containing the minimum information as specified in Schedule 3and shall be made readily available for inspection for period as specified.

### Fees

- The fees which may be levied for survey, inspection and certification under these rules shall be as specified in the Schedule 4.

### Penalty

- Whoever contravenes any provisions of these rules shall be punished in accordance with the provisions contained under sub section (2) of section XXX of the Act.

# Annex

# Regulations for the Control and Management of Ships' Ballast Water and Sediments

## Section A - General Provisions

### Regulation A-1: Definitions

For the purposes of this Annex:

- "Anniversary date" means the day and the month of each year corresponding to the date of expiry of  the Certificate.

- "Ballast Water Capacity" means the total volumetric capacity of any tanks, spaces or compartments on a ship used for carrying, loading or discharging Ballast Water, including any multi-use tank, space or compartment designed to allow carriage of Ballast Water.

- "Company" means the owner of the ship or any other organization or person such as the manager, or the bareboat charterer, who has assumed the responsibility for operation of the ship from the owner of the ship and who on assuming such responsibility has agreed to take over all the duties and responsibilities imposed by the International Safety Management Code*.

*Refer to the ISM Code adopted by the IMO by resolution A.741(18),as amended.

- "Constructed" in respect of a ship means a stage of construction where:

  - the keel is laid; or

  - construction identifiable with the specific ship begins;

  - assembly of the ship has commenced comprising atleast50tonnesor1percentof the estimated mass of all structural material, whichever is less; or

  - the ship undergoes a major conversion.

- "Major conversion" means a conversion of a ship:

  - which changes its ballast water carrying capacity by15 percent or greater, or

  - which changes the ship type, or

  - which, in the opinion of the Administration, is projected to prolong its life by ten years or more, or

  - which results in modifications to its ballast water system other than component replacement-in-kind. Conversion of a ship to meet the provisions of regulation D- 1shallnotbedeemed to constitute a major conversion for the purpose of this Annex.

- "Active Substance" means a substance or organism, including a virus or a fungus that has a general or specific action on or against Harmful Aquatic Organisms and Pathogens.

### Regulation A-2: General Applicability

Except where expressly provided otherwise, the discharge of Ballast Water shall only be conducted through Ballast Water Management in accordance with the provisions of this Annex.

### Regulation A-3: Exceptions

The requirements of regulation B-3, or any measures adopted by Government of India pursuant to Article 2.3 and Section C, shall not apply to:

- the uptake or discharge of Ballast Water and Sediments necessary for the purpose of ensuring the safety of a ship in emergency situations or saving life at sea; or

- the accidental discharge or ingress of Ballast Water and Sediments resulting from damage to a ship or its equipment:

  - provided that all reasonable precautions have been taken before and after the occurrence of the damage or discovery of the damage or discharge for the purpose of preventing or minimizing the discharge; and

  - unless the owner, Company or officer in charge willfully or recklessly caused damage; or

- the  uptake and  discharge of  Ballast  Water and  Sediments  when  being  used  for  the purpose of avoiding or minimizing pollution incidents from the ship; or

- the uptake and subsequent discharge on the high seas of the same Ballast Water and Sediments; or

- the discharge of Ballast Water and Sediments from a ship at the same location where the whole of that Ballast Water and those Sediments originated and provided that no mixing with unmanaged Ballast Water and Sediments from other areas has occurred. If mixing has occurred, the Ballast Water taken from other areas is subject to Ballast Water Management in accordance with this Annex.

### Regulation A-4: Exemptions

- At the decision of Government of India, in waters under its jurisdiction, may grant exemptions to any requirements to apply regulations B-3 or C-1, in addition to those exemptions contained elsewhere in this rule, but only when they are:

  - granted to a ship or ships on a voyage or voyages between specified ports or locations; or to a ship which operates exclusively between specified ports or locations;

  - effective for a period of no more than five years subject to intermediate review;

  - granted to ships that do not mix Ballast Water or Sediments other than between the ports or locations specified in paragraph 1.1; and

  - granted based on the Guidelines on risk assessment developed bythe IMO.

- Exemptions granted pursuant to paragraph 1 shall not be effective until after communication to the Organization and circulation of relevant information to the Parties.

- Any exemptions granted under this regulation shall not impair or damage the environment, human health, property or resources of adjacent or other country.

- Any exemptions granted under this regulation shall be recorded in the Ballast Water record book.

### Regulation A-5: equivalent compliance

Equivalent compliance with this Annex for pleasure craft used solely for recreation or competition or craft used primarily for search and rescue, less than 50 metres in length overall, and with a maximum Ballast Water capacity of 8 cubic metres shall be determined by the Directorate General of Shipping on a case by case basis.

## SECTION B - MANAGEMENT AND CONTROL REQUIREMENTS FOR SHIPS

### Regulation B-1: ballast water management plan

Each ship shall have on board and implement a Ballast Water Management plan. The Ballast Water Management plan shall be specific to the ship and at least:

- Detail safety procedures for the ship and the crew associated with Ballast Water Management as required by these rules;

- Provide a detailed description of the actions to be taken to implement the Ballast Water Management requirements and supplemental Ballast Water Management practices as set forth in therules;

- detail the procedures for the disposal of Sediments:

  - at sea; and

  - to shore;

- Include the procedures for coordinating shipboard Ballast Water Management that involves discharge to the sea with the authorities of the State in to whose waters such discharge will take place;

- Designate the officer onboard in charge of ensuring that the plan is properly implemented;

- contain the reporting requirements for ships provided for under Convention.

### Regulation B-2: Ballast water record book

- Each ship shall have onboard a Ballast Water record book that may be an electronic record system, or that may be integrated into another record book or system and, which shall at least contain the information specified in Schedule 3.

- Ballast Water record book entries shall be maintained onboard the ship for a minimum period of two years after the last entry has been made and thereafter in the Company’s control for a minimum period of three years.

- In the event of the discharge of Ballast WaterpursuanttoregulationsA-3, A-4 or B-3.6orinthe event of other accidental or exceptional discharge of Ballast Water not otherwise exempted by these rules, an entry shall be made in the Ballast Water record book describing the circumstances of, and the reason for, the discharge.

- The Ballast Water record book shall be kept readily available for inspection at all reasonable times and, in the case of an unmanned ship under tow, may be kept on the towing ship.

- Each operation concerning Ballast Water shall be fully recorded without delay in the Ballast Water record book. Each entry shall be signed by the officer in charge of the operation concerned and each completed page shall be signed by the master.

- The Surveyors of Director General of shipping or organization recognized by it may inspect the Ballast Water record book on board any ship to which these rules apply while the ship is in its port or offshore terminal, and may make a copy of any entry, and require the master to certify that the copy is a true copy. Any copy so certified shall be admissible in any judicial proceeding as evidence of the facts stated in the entry.

### Regulation B-3: Ballast Water Management for Ships

- A ship constructed before2009:

  - With a Ballast Water Capacity of between 1,500 and5,000 cubic metres, inclusive, shall conduct Ballast Water Management that at least meet the standards described in regulationD-1 or regulation D-2 until 2014,after which time it atleast meet the standard described in regulation D-2;

  - With a Ballast Water Capacity of less than1,500 or greaterthan5,000 cubic metres shall conduct Ballast Water Management that at least meets the standard described in regulation D-1 or regulation D-2 until 2016,after which time it shall at least meet the standard described in regulation D-2.

- A ship to which paragraph 1 applies shall comply with paragraph 1 not later than the first intermediate or renewal survey, whichever occurs first, after the anniversary date of delivery of the ship in the year of compliance with the standard applicable to the ship.

- A ship constructed in or after2009witha Ballast Water Capacityoflessthan5,000 cubic metres shall conduct Ballast Water Management that at least meets the standards described in regulation D-2.

- A ship constructed in or after2009,but before 2012,witha Ballast Water Capacity of 5,000 cubic metres or more shall conduct Ballast Water Management in accordance with paragraph 1.2.

- A ship constructed in or after2012 with a Ballast Water Capacity of 5000 cubic metres or more shall conduct Ballast Water Management that at least meets the standard described in regulation D-2.

- The requirements of this regulation do not apply to ships that discharge Ballast Water to a reception facility designed taking into account the Guidelines developed by the IMO for such facilities.

- Other methods of Ballast Water Management may also be accepted as alternatives to the requirements described in paragraphs 1 to 5, provided that such methods ensure at least the same level of protection to the environment, human health, property or resources, and are approved in principle by the MEPC.

### Regulation B-4: Ballast Water Exchange

- A ship conducting Ballast Water exchange to meet the standard in regulation D-1 shall:

  - whenever possible, conduct such Ballast Water exchange at least 200 nautical miles from the nearest land and in water at least 200 metres in depth, taking into account the Guidelines developed by the IMO;

  - in cases where the ship is unable to conduct Ballast Water exchange in accordance with paragraph 1.1, such Ballast Water exchange shall be conducted taking into account the Guidelines described in paragraph 1.1 and as far from the nearest land as possible, and in all cases at least 50 nautical miles from the nearest land and in water at least 200 metres in depth.

- In sea areas where the distance from the nearest land or the depth does not meet the parameters described in paragraph 1.1 or 1.2,thePortState may designate areas, in consultation with adjacent to other countries, as appropriate, where a ship may conduct Ballast Water exchange, taking into account the Guidelines described in paragraph 1.1.

- A ship shall not be required to deviate from its intended voyage, or delay the voyage, in order to comply with any particular requirement of paragraph 1.

- A ship conducting Ballast Water exchange shall not be required to comply with paragraphs1or2, as appropriate, if the master reasonably decides that such exchange would threaten the safety or stability of the ship, its crew, or its passengers because of adverse weather, ship design or stress, equipment failure, or any other extraordinary condition.

- When a ship is required to conduct Ballast Water exchange and does not do so in accordance with this regulation, the reasons shall be entered in the Ballast Water record book.

### Regulation B-5: Sediment Management for Ships

- All ships shall remove and dispose off Sediments from spaces designated to carry Ballast Water in accordance with the provisions of the ship’s Ballast Water Management plan.

- Ships described in regulation B-3.3 to B-3.5 should, without compromising safety or operational efficiency, be designed and constructed with a view to minimize the uptake and undesirable entrapment of Sediments, facilitate removal of Sediments, and provide safe access to allow for Sediment removal and sampling, taking into account guidelines developed by the IMO. Ships described in regulation B-3.1 should, to the extent practicable, comply with this paragraph.

### Regulation B-6: Duties of Officers and Crew

Officers and crew shall be familiar with their duties in the implementation of Ballast Water Management particular to the ship on which they serve and shall, appropriate to their duties, be familiar with the ship's Ballast Water Management plan.

## SECTION C - SPECIAL REQUIREMENTS IN CERTAIN AREAS

### Regulation C-1: Additional Measures

- Government of India, individually or jointly with other countries which are party to the Convention, if determines that measures in addition to those in SectionB are necessary to prevent, reduce, or eliminate the transfer of Harmful Aquatic Organisms and Pathogens through ships' Ballast Water and Sediments, may specify additional standards or requirements, consistent with international law, in a manner as may be specified time to time.

### Regulation C-2: Warnings Concerning Ballast Water Uptake in Certain Areas and Related Flag State Measures

- Director General or competent authority as may be specified, will notify mariners of areas under its jurisdiction where ships should not uptake Ballast Water due to known conditions. Warnings may be issued for areas:

  - Known to contain out breaks, infestations, or populations of Harmful Aquatic Organisms and Pathogens(e.g., toxic algalblooms) which are likely to be of relevance to Ballast Water uptake or discharge;

  - near sewage outfalls; or

  - Where tidal flushing is poor or times during which a tidal stream is known to be more turbid.

## SECTION D - STANDARDS FOR BALLAST WATER MANAGEMENT

### Regulation D-1: Ballast Water Exchange Standard

- Ships performing Ballast Water exchange in accordance with this regulation shall do so with an efficiency of at least 95 percent volumetric exchange of Ballast Water.

- For ships exchanging Ballast Water by the pumping-through method, pumping through three times the volume f each Ballast Water tank shall be considered to meet the standard described in paragraph 1.Pumpingthroughlessthanthreetimesthevolumemaybe accepted provided the ship can demonstrate that at least 95percent volumetric exchange is met.

### Regulation D-2: Ballast Water Performance Standard

- Ships conducting Ballast Water Management in accordance with this regulation shall discharge less than 10 viable organisms per cubic metre greater than or equalto50micrometres in minimum dimension and less than10 viable organisms per milliliter less than 50 micro metres in minimum dimension and greater than or equal to10micrometresinminimumdimension; and discharge of the indicator microbes shall not exceed the specified concentrations described in paragraph 2.

- Indicator microbes, as a human health standard, shall include:

  - Toxicogenic Vibriocholerae (O1 andO139)withlessthan1 colony forming unit (cfu)per100millilitresorlessthan1cfu per1 gram(wet weight) zooplankton samples ;

  - Escherichia coli less than250 cfu per 100millilitres;

  - Intestinal Enterococci less than 100 cfu per 100milliliters.

### Regulation D-3: Approval requirements for Ballast Water Management systems

- Except as specified in paragraph2, Ballast Water Management systems used to comply with these rules must be approved by an Administration or an organization recognized on its behalf ,taking into account the Guidelines developed by the IMO.

- Ballast Water Management systems which make use of Active Substances or preparations containing one or more Active Substances to comply with the Convention shall be approved by the IMO, based on a procedure developed by the IMO. This procedure shall describe the approval and withdrawal of approval of Active Substances and their proposed manner of application. At withdrawal of approval, the use of the relevant Active Substance or Substances shall be prohibited within 1 year after the date of such withdrawal.

- Ballast Water Management systems used to comply with this Convention must be safe in terms of the ship, its equipment and the crew.

### Regulation D-4: Prototype Ballast Water Treatment Technologies

- For any ship that, prior to the date that the standard in regulation D-2 would otherwise become effective for it, participates in a programme approved by the Director General, to test and evaluate promising Ballast Water treatment technologies, the standard in regulationD-2shallnot apply to that ship until five years from the date on which the ship would otherwise be required to comply with such standard.

- For any ship that, after the date on which the standard in regulation D-2hasbecome effective for it, participates in a programme approved by the Administration, taking into account Guidelines developed by the IMO, to test and evaluate promising Ballast Water technologies with the potential to result in treatment technologies achieving a standard higher than that in regulation D-2 the standard in regulation D-2shall cease to apply to that ship for five years from the date of installation of such technology.

- Throughout the test and evaluation period, the treatment system must be operated consistently and as designed.

## SECTION E - SURVEY AND CERTIFICATION REQUIREMENTS FOR BALLAST WATER MANAGEMENT

### Regulation E-1: Surveys

- Ships of 400 gross tonnage and above, to which this regulation applies, and engaged in international voyages or plying along the coast of India, excluding floating platforms, FSUs and FPSOs, shall be subject to surveys as specified below:

  - an initial survey to verify that the Ballast Water Management plan and any associated structure, equipment, systems, fitting, arrangements and material or processes comply fully with the requirements of the Convention before the ship is put in service or before the International Ballast Water Certificate or, as the case may be, the Indian Ballast Water Certificate is issued for the first time; and

  - an additional survey either general or partial, after a change, replacement, or significant repair of the structure, equipment, systems, fittings, arrangements and material necessary to achieve full compliance with these rules and accordingly there is change or replacement in the ballast water management plan and such change or replacement has been endorsed on the International Ballast Water Management Certificate or, as the case may be, the Indian Ballast Water Management Certificate; and

  - a renewal survey at intervals not exceeding five years; and

  - an intermediate survey within three months before or after the second anniversary date or within three months before or after the third anniversary date of the Certificate, which shall take the place of one of the annual surveys; and

  - an annual survey within three months before or after each anniversary date, including a general inspection of the structure, any equipment, systems, fittings, arrangements and material or processes associated with the Ballast Water Management plan to ensure that they have been maintained to remain fit in all respects to proceed to sea without presenting a threat of harm to the environment, human health, property or resources and remain satisfactory for the service for which the ship is intended.

  - If, on report from the surveyor or other person authorized to inspect a ship, the Director General is satisfied that any provision     of the Convention has not been complied with or contravened by such ship within the coastal waters, the Director General or any officer authorized by him in that behalf, may-

- (i) detain the ship until the causes of such non –compliance or contravention are removed to the satisfaction of the Director General or the officer authorized by him; and

- Provided that where the Director Generaldeems it necessary, he may request the Indian Navy or the Coast Guard for preventing the ship from proceeding to sea, and the Indian Navy or the Coast Guard, as the case may be, shall take action as requested by the Director-General.

(ii) On receipt of information from the Government of any country to which the Convention applies that an Indian ship or other ship has contravened any provision of the Convention, the Central Government may, if it deems fit necessary, request such Government to furnish further details of the alleged contravention and if satisfied that sufficient evidence is available, conduct investigation of the alleged violations and take appropriate measures inrespect thereof.

- The surveys referred to in sub-rule (1).shall be such as to ensure that the ship's ballast water management plan is in accordance with and fully complies with the provisions of these rules.

- The surveys of the ships referred to in sub-rule (1).shall be carried out by officers authorized by the Director General or by recognized organizations.

- The Director General shall authorize the surveyor or the recognized organizations to conduct surveys so as to -

  - verify whether the ships comply with the provisions of rule 3; and

  - carry out surveys, if requested by the appropriate authorities of a port State that is a Party to the Convention.

- When the Director General or a recognized organization determines that the ship's Ballast water management plan does not confirm either to the particulars of the International Ballast Water Management Certificate, or the Indian Ballast Water Management Certificate, or to the provisions of the Act or these rules, the surveyor or the recognized organization shall immediately ensure that corrective action is taken to bring the ship into compliance, and the surveyor or the recognized organization shall notify the Director General of any such determination.

- If the corrective action referred to in Sub rule (5). is not complied with, the Director General shall be informed in writing by the surveyor or the recognized organization forthwith and the International Ballast Water Management Certificate or the Indian Ballast Water Management Certificate shall not be issued or if already issued, the respective certificate shall, by order, be withdrawn by the Director General.

- In a situation specified in clause(c) of sub rule (3), if the ship is in the port of another Party, the appropriate authorities of the port State and Director General shall be informed forthwith in writing by the surveyor or recognized organization.

### Regulation E-2: Issue or endorsement of an International Ballast Water Management Certificate or Indian Ballast Water Management Certificate

- The Director General or a recognized organization shall require, after the successful completion of a survey in accordance with rule 5, to issue or endorse:

  - International Ballast Water Management Certificate to an Indian ship or other ships entitled to fly Indian flag or operating under its authority, which is of 400 gross tonnage and above and engaged in International voyage; and

  - Indian Ballast Water Management Certificate to an Indian ship entitled to fly Indian flag, which are of 400 gross tonnage and above, with appropriate conditions as applicable for each type of ships and not engaged in international voyage and are required to be registered under this Act, as the case may be.

### Regulation E-3: Issue or Endorsement of an International Ballast Water Management Certificate by Director General or another country which is party to Convention

- At the request of the Administration, the Government of a country to which the Convention applies or vice versa, may cause a ship to be surveyed and, if satisfied, shall issue or authorize the issue of the International Ballast Water Management Certificate to the ship and, where appropriate, endorse or authorize the endorsement of the International Ballast Water Management Certificate for the ship in accordance with the Convention.

- Upon complying with sub-rule (1) above, a copy of the Certificate and a copy of the survey report shall be transmitted as soon as possible to the Administration, or to the Government of another country which is party to the Convention, as the case may be.

- The certificate so issued shall contain a statement that it has been issued at the request, of the Administration, or of the Government of another country which is party to the Convention, as the case may be.

### Regulation E-4: Form of the Certificate

- The Certificate and record shall be drawn up in the form corresponding to the model given in schedule 1 or as the case may be, schedule 2, appended to these rules.

### Regulation E-5: Duration and Validity of the Certificate

- An International Ballast Water Management Certificate or an Indian Ballast Water Management Certificate may be issued by the Director General or organization recognized by it, for a period not exceeding five years.

- For renewal surveys:

  - Notwithstanding the requirements of paragraph(1) above, when the renewal survey is completed within three months before the expiry date of the existing Certificate, the new Certificate shall be valid from the date of completion of the renewal survey to a date not exceeding five years from the date of expiry of the existing Certificate.

  - When the renewal survey is completed after the expiry date of the existing Certificate, the new Certificate shall be valid from the date of completion of the renewal survey to a date not exceeding five years from the date of expiry of the existing Certificate.

  - When the renewal survey is completed more than three months before the expiry date of the existing Certificate, the new Certificate shall be valid from the date of completion of the renewal survey to a date not exceeding five years from the date of completion of the renewal survey.

- If  a Certificate is issued for a period of less than five years, the Director General or organization recognized by it may extend the validity of the Certificate beyond the expiry date to the maximum period specified in paragraph(1)., provided that the surveys referred to in regulation E-1.(1).d. applicable when a Certificate is issued for a period of five years are carried out as appropriate.

- If a renewal survey has been completed and a new Certificate cannot be issued or placed onboard the ship before the expiry date of the existing Certificate, the person or organization authorized by the Director General may endorse the existing Certificate and such a Certificate shall be accepted as valid for a further period which shall not exceed five months from the expiry date.

- If a ship at the time when the Certificate expires is not in a port in which it is to be surveyed, the Director General or organization recognized by it, may extend the period of validity of the Certificate but this extension shall be granted only for the purpose of allowing the ship to complete its voyage to the port in which it is to be surveyed, and then only in cases where it appears proper and reasonable to do so. No Certificate shall be extended for a period longer than three months, and a ship to which such extension is granted shall not, on its arrival in the port in which it is to be surveyed, be entitled by virtue of such extension to leave that port with out having a new Certificate. When the renewal survey is completed, the new Certificate shall be valid to a date not exceeding five years from the date of expiry of the existing Certificate before the extension was granted.

- A Certificate issued to a ship engaged on short voyages which has not been extended under the foregoing provisions of this regulation maybe extended by the Director General or organization recognized by it, for a period of grace of up to one month from the date of expiry stated on it. When the renewal survey is completed, the new Certificate shall be valid to a date not exceeding five years from the date of expiry of the existing Certificate before the extension was granted.

- In special circumstances, as determined by the Director General or organization recognized by it, a new Certificate need not be dated from the date of expiry of the existing Certificate as required by paragraph(2).b ,(5).or(6).of this regulation. In these special circumstances, the new Certificate shall be valid to a date not exceeding five years from the date of completion of the renewal survey.

- If an annual survey is completed before the period specified in regulation E-1, then:

  - The Anniversary date shown on the Certificate shall be amended by endorsement toadatewhichshallnotbemorethanthreemonthslaterthanthedateonwhich the survey was completed;

  - The subsequent annual or intermediate survey required by regulation E-1 shall be completed at the intervals prescribed by that regulation using the new Anniversary date;

  - the expiry date may remain unchanged provided one or more annual surveys, as appropriate, are carried out so that the maximum intervals between the surveys prescribed by regulation E-1 are not exceeded.

- A Certificate issued under regulation E-2or E-3shall cease to be valid in any of the following cases:

  - If the structure, equipment, systems, fittings, arrangements and material necessary to comply fully with this Convention is changed, replaced or significantly repaired and the Certificate is not endorsed in accordance with this Annex;

  - Up on transfer of the ship to the flag of another State. A new Certificate shall only be issued when the Party issuing the new Certificate is fully satisfied that the ship is in compliance with the requirements of regulation E-1. In the case of a transfer between Parties, if requested within three months after the transfer has taken place, the Party whose flag the ship was formerly entitled to fly shall, as soon as possible, transmit to the Administration copies of the Certificates carried by the ship before the transfer and, if available, copies of the relevant survey reports;

  - If the relevant surveys are not completed within the periods specified under regulation E-1.1; or

  - If the Certificate is not endorsed in accordance with regulation E-1.1.

## SCHEDULE 1

[Seesub-rule(2)oftherule 5]

## INTERNATIONAL BALLAST WATER MANAGEMENT CERTIFICATE

Issued under the provisions of the International Convention for the Control and Management of Ships' Ballast Water and Sediments (hereinafter referred to as "the Convention") under the authority of the Government of India

by

(full designation of the competent person or organization  authorized under the provisions of the Convention)

Particulars of ship

Name of ship

Distinctive number or letters

Port of registry

Gross Tonnage

IMO number"

Date of Construction

Ballast Water Capacity (in cubic metres)

Details of Ballast Water Management Method(s) Used

Method of Ballast Water Management used

Date installed (if applicable)

Name of manufacturer (if applicable)

The principal Ballast Water Management method(s) employed on this ship is/are:

in accordance with regulation D-l

in accordance with regulation D-2

(describe)

the ship is subject to regulation D-4

THIS IS TO CERTIFY:

- That the ship has been surveyed in accordance with regulation E-1 of the Annex to the Convention: and

- That the survey shows that Ballast Water Management on the ship complies with the Annex to the Convention.

This certificate is valid until 	subject to surveys in accordance with

regulation E-1 of the Annex to the Convention.

Completion date of the survey on which this certificate is based: dd/ mm/ yyyy

Issued at

(Place of issue of certificate)

{Date of issue)	Signature of authorized official issuing the certificate)

(Seal or stamp of the authority, as appropriate)

ENDORSEMENT FOR ANNUAL AND INTERMEDIATE SURVEY(S)

THIS IS TO CERTIFY that a survey required by regulation E-1 of the Annex to the Convention the ship was found to comply with the relevant provisions of the Convention:

Annual survey:	Signed

(Signature of duly authorized official)

Place

Date

(Seal or stamp of the authority, as appropriate)

Annual* /Intermediate survey*:	Signed

(Signature of duly authorized official)

Place

Date

(Seal or stamp of the authority, as appropriate)

Annual*/ Intermediate survey*:	Signed

(Signature of duly authorized official)

Place

Date

(Seal or stamp of the authority, as appropriate)

Annual survey:	Signed

(Signature of duly authorized official)

Place

Date

(Seal or stamp of the authority, as appropriate)

* Delete as appropriate.

ANNUAL/INTERMEDIATE SURVEY IN

ACCORDANCE WITH REGULATION E-5.8.3

THIS IS TO CERTIFY that, at an annual/intermediate* survey in accordance with regulation E-5.8.3 of the Annex to the Convention, the ship was found to comply with the relevant provisions of the Convention:

Signed

(Signature of authorized official)
Place

Date

(Seal or stamp of the authority, as appropriate)

ENDORSEMENT TO EXTEND THE CERTIFICATE IF VALID FOR LESS THAN 5 YEARS WHERE REGULATION E-5.3APPLIES

The ship complies with the relevant provisions of the Convention, and this Certificate shall, in
accordance with regulation E-5.3 of the Annex to the Convention, be accepted as valid
until

Signed

(Signature of authorized official)

Place

Date

(Seal or stamp of the authority, as appropriate)

* Delete as appropriate

ENDORSEMENT WHERE THE RENEWAL SURVEY HAS BEEN COMPLETED AND REGULATION E-5.4APPLIES

The ship complies with the relevant provisions of the Convention and this Certificate shall, in accordance with regulation E-5.4 of the Annex to the Convention, be accepted as valid until

Signed

(Signature of authorized official)

Place

Date

(Seal or stamp of the authority, as appropriate)

ENDORSEMENT TO EXTEND THE VALIDITY OF THE CERTIFICATE UNTIL

REACHING THE PORT OF SURVEY OR FOR A PERIOD OF GRACE

WHERE REGULATION E-5.5 OR E-5.6APPLIES

Tins Certificate shall,  in accordance with regulation E-5.5 or E-5.6*   of the Annex to the
Convention, be accepted as valid until

Signed

(Signature of authorized official)

Place

Date

(Seal or stamp of the authority, as appropriate)

ENDORSEMENT FOR ADVANCEMENT OF ANNIVERSARY DATE WTIERE REGULATION E-5.8 APPLIES

In accordance with regulation E-5.S of tlie Annex to the Convention the new Anniversary date is

Signed

(Signature of authorized official)

Place

Date

(Seal or stamp of the authority, as appropriate)

In accordance with regulation E-5.8 of tlie Annex to the Convention the new Anniversary date is

Signed

(Signature of duly authorized official)

Place

Date

(Seal or stamp of the authority, as appropriate)

* Delete as appropriate

## SCHEDULE 2

[Seesub-rule(2)oftherule 5]

## INDIANBALLAST WATER MANAGEMENTCERTIFICATE

Issued under Merchant Shipping(Control and Management of Ships’ Ballast Water and sediments) Rules, 2017 on Ships under the authority of the Government of India

by

(full designation of the competent person or organization  authorized)

Particular's of ship

Name of ship

Distinctive number or letters

Port of registry

Gross Tonnage

IMO number

Date of Construction

Ballast Water Capacity (in cubic metres)

Details of Ballast Water Management Method(s) Used

Method of Ballast Water Management used

Date installed (if applicable)

Name of manufacturer (if applicable)

The principal Ballast Water Management method(s) employed on this ship is/are:

in accordance with regulation D-l of the Annex

in accordance with regulation D-2of the Annex

(describe)

the ship is subject to regulation D-4 of the Annex

THIS IS TO CERTIFY THAT:

- The ship has been surveyed in accordance with rule X of the Merchant Shipping (Control and Management of Ships’ Ballast Water and Sediments)Rules,2017;and

- The survey shows that Ballast Water Management System on the ship complies with the applicable requirements of Merchant Shipping Control and Management of Ships’ Ballast Water and Sediments)Rules,2017

Completion date of the survey on which this certificate is based: dd/ mm/ yyyy

Issued at

(Place of issue of certificate)

{Date of issue)	Signature of authorized official issuing the certificate)

(Seal or stamp of the authority, as appropriate)

ENDORSEMENT FOR ANNUAL AND INTERMEDIATE SURVEY(S)

THIS IS TO CERTIFY that a survey required by regulation E-1 of the Annex to the Merchant Shipping(Control and Management of Ships’ Ballast Water and Sediments)Rules,2017, and  ship was found to comply with the relevant provisions of the rules:

Annual survey:	Signed

(Signature of duly authorized official)

Place

Date

(Seal or stamp of the authority, as appropriate)

Annual*/ Intermediate survey*:	Signed

(Signature of duly authorized official)

Place

Date

(Seal or stamp of the authority, as appropriate)

Annual* /Intermediate survey*:	Signed

(Signature of duly authorized official)

Place

Date

(Seal or stamp of the authority, as appropriate)

Annual survey:	Signed

(Signature of duly authorized official)

Place

Date

(Seal or stamp of the authority, as appropriate)

* Delete as appropriate.

ANNUAL/INTERMEDIATE SURVEY IN ACCORDANCE WITH REGULATION E-5.8.3

THIS IS TO CERTIFY that, at an annual/intermediate* survey in accordance with regulation E-5.8.3 of the Annex to the Merchant Shipping(Control and Management of Ships’ Ballast Water and Sediments)Rules,2017, and the ship was found to comply with the relevant provisions of the rules:

Signed

(Signature of authorized official)
Place

Date

(Seal or stamp of the authority, as appropriate)

ENDORSEMENT TO EXTEND THE CERTIFICATE IF VALID FOR LESS THAN 5 YEARS WHERE REGULATION E-5.3APPLIES

The ship complies with the relevant provisions of the Merchant Shipping (Control and Management of Ships’ Ballast Water and Sediments)Rules, 2017, and this Certificate shall, in
accordance with regulation E-5.3 of the Annex to the rules, be accepted as valid
until

Signed

(Signature of authorized official)

Place

Date

(Seal or stamp of the authority, as appropriate)

* Delete as appropriate

ENDORSEMENT WHERE THE RENEWAL SURVEY HAS BEEN COMPLETED AND REGULATION E-5.4APPLIES

The ship complies with the relevant provisions of the Convention and this Certificate shall, in accordance with regulation E-5.4 of the Annex to the Merchant Shipping (Control and Management of Ships’ Ballast Water and Sediments)Rules, 2017, be accepted as valid until

Signed

(Signature of authorized official)

Place

Date

(Seal or stamp of the authority, as appropriate) " Delete as appropriate

ENDORSEMENT TO EXTEND THE VALIDITY OF THE CERTIFICATE UNTIL

REACHING THE PORT OF SURVEY OR FOR A PERIOD OF GRACE

WHERE REGULATION E-5.5 OR E-5.6 APPLIES

This Certificate shall, in accordance with regulation E-5.5 or E-5.6   of the Annex to the Merchant Shipping(Control and Management of Ships’ Ballast Water and Sediments)Rules,2017, be accepted as valid until

Signed

(Signature of authorized official)

Place

Date

(Seal or stamp of the authority, as appropriate)

ENDORSEMENT FOR ADVANCEMENT OF ANNIVERSARY DATE WHERE REGULATION E-5.8APPLIES

In accordance with regulation E-5.8 of the Annex to the Merchant Shipping (Control and Management of Ships’ Ballast Water and Sediments)Rules,2017, and the new Anniversary date is

Signed

(Signature of authorized official)

Place

Date

(Seal or stamp of the authority, as appropriate)

In accordance with regulation E-5.8 of the Annex to the Merchant Shipping (Control and Management  of Ships’ Ballast Water and Sediments)Rules,2017, and the new Anniversary date is

Signed

(Signature of duly authorized official)

Place

Date

(Seal or stamp of the authority, as appropriate)

*Delete as appropriate

## SCHEDULE 3

[See sub-rule(2)of the rule 8]

## Form of ballast water record book

INTERNATIONAL CONVENTION FOR THE CONTROL AND MANAGEMENT OF SHIPS' BALLAST WATER AND SEDIMENTS

Period From: .......................................... To: ..................................................................................

Name of Ship ...................................................................................................................................

IMO number .....................................................................................................................................

Gross tonnage....................................................................................................................................

Flag................................................................................................................................................... Total Ballast Water capacity (in cubic metres) .................................................................................

The ship is provided with a Ballast Water Management plan Diagram of ship indicating ballast tanks:

- Introduction

In accordance with regulation B-2 of the Annex to the International Convention for the Control and Management of Ships' Ballast Water and Sediments, a record is to be kept of each Ballast Water operation. This includes discharges at sea and to reception facilities.

- Ballast Water and Ballast Water Management

"Ballast Water" means water with its suspended matter taken onboard a ship to control trim, list, draught, stability, or stresses of a ship. Management of Ballast Water shall be in accordance with an approved Ballast Water Management plan and taking in to account Guidelines*developed by the Organization.

- Refer to the Guidelines for the control and management of ships 'ballast water to minimize the transfer of harmful aquatic organisms and pathogens adopted by the Organization by resolution A.868(20).

- Entries in the Ballast Water Record Book

Entries in the Ballast Water record book shall be made on each of the following occasions:

  - When Ballast Water is taken on board:

    - Date, time and location of port or facility of uptake(port or lat/long), depth if outside port

    - Estimated volume of uptake in cubic metres

    - Signature of the officer in charge of the operation.

  - Whenever Ballast Water is circulated or treated for Ballast Water Management purposes:

- Date and time of operation

- Estimated volume circulated or treated (in cubic metres)

- Whether conducted in accordance with the Ballast Water Management plan

- Signature of the officer in charge of the operation

  - When Ballast Water is discharged into the sea:

- Date, time and location port or facility of discharge (port or lat/long)

- Estimated volume discharged in cubic metres plus remaining volume in cubic metres

- Whether approved Ballast Water Management plan had been implemented prior to discharge

- Signature of the officer in charge of the operation.

  - When Ballast Water is discharged to a reception facility:

- Date, time, and location of uptake

- Date, time, and location of discharge

- Port or facility

- Estimated volume discharged or taken up, in cubic metres

- Whether approved Ballast Water Management plan had been implemented prior to discharge

- Signature of officer in charge of  the operation

  - Accidental or other exceptional up take or discharges of Ballast Water:

- Date and time of occurrence

- Port or position of the ship at time of occurrence

- Estimated volume of Ballast Water discharged

- Circumstances of uptake, discharge, escape or loss, the reason therefore and general remarks.

- Whether approved Ballast Water Management plan had been implemented prior  to discharge

- Signature of officer in charge of the operation

  - Additional operational procedure and general remarks

Volume of Ballast Water

The volume of Ballast water onboard should be estimated in cubic metres. The Ballast water record book contains many references to estimated volume of Ballast Water. It is recognized that the accuracy of estimating volumes of ballast is left to interpretation.

RECORD OF BALLAST WATER OPERATIONS

SAMPLE BALLAST WATER RECORD BOOKPAGE

Name of Ship:................................................................................................................................

Distinctive number or letters ...........................................................................................................

| Date | Item (number) | Record of operations/signature of officers in charge |
| --- | --- | --- |

Signature of master................................................................

## SCHEDULE 4

[Seerule 18]

## Fees

| SR. No. | ITEMS | CATEGORY | Fees |
| --- | --- | --- | --- |
| 1. | Survey to verify compliance with the provisions of Merchant Shipping (Ballast water Management ), Rules 2017. | a) Initial Survey (i) Vessel’s Between 400 to 1000 GT (ii) Vessel’s above 1000 GT b) Renewal Survey (i) Vessel’s Between 400 to 1000 GT (ii) Vessel’s above 1000 GT b) Additional Survey (i) Vessel’s Between 400 to 1000 GT (ii) Vessel’s above 1000 GT c) Periodical Survey (Annual Survey/ Intermediate Survey) | Rs.10,000/- Rs.10,000/- Plus Rs.1,000/- for every additional 500 GT or part thereof subject to maximum of Rs.40,000/- Rs.8,000/- Rs.8,000/- plus Rs.5,00/- for every additional 500 GT or Part thereof subject to maximum of Rs.35,000/- Rs.8,000/- Rs.8,000/- plus Rs.5,00/- for every additional 500 GT or Part thereof subject to maximum of Rs.35,000/- 75% of renewal survey fees subject to a min of Rs.6,500/- |
| 2 | Issuance of certificate |  | Rs.3,000/- |
| 3 | Permissions in respect of extension of surveys/ exemptions/ dispensation etc. |  | Rs.1,500/- perCase |
| 4 | Approval of plans related to Ballast water management system |  | Rs.8000/- |